Link budget flow chart adding:

Mission phase

↓

Define communication link

↓

Define range and visibility

↓

Define required data rate

↓

Choose frequency band

↓

Choose transmitter power and antenna gain

↓

Calculate free-space path loss

↓

Add other losses

↓

Choose receiver antenna / ground station / G/T

↓

Calculate received power

↓

Calculate C/N0

↓

Calculate Eb/N0

↓

Compare with required Eb/N0

↓

Calculate link margin

↓

Feasible?

↓

Yes → save design

No → change architecture / antenna / power / data rate / coding

Standard link budget structure: Pr = Pt + Gt + Gr − Lt − LFS − Lr – Lmisc

Start from the small model start with each L1, L2 and L3 : uplink, downlink and relay. For each link, calculate:

FSPL

EIRP

Received power

C/N0

Eb/N0

Link margin

This architecture is feasible if:

- Earth link uses X-band/S-band

- relay has antenna gain around ___ dBi

- lander transmit power is around ___ W

- emergency link data rate is reduced to ___ bps/kbps

- ground station gain is around ___ dBi

TTC: communication and control link with humans on Earth

this is mostly used in uplink and downlink.

Uplink includes all the command  from G to S.

Downlink : all signals from S to G, telemetry.

Tracking: where the spacecraft is,

Range measurement:  which calculates the distance of the lander

Doppler measurement: the speed of the spacecraft, trajectory

Angle measurement: thrust control

Commands for example:

Start engine burn

Stop engine burn

Open valve

Change attitude

Switch antenna

Start video transmission

Enter safe mode

Upload new software command sequence

Prepare for ascent

Prepare for Earth return

Mission process:

TTC requirement: commands

↓

Required data rate and reliability: data modulation

↓

Link budget calculation: make sure commands can arrive & telemetry can be received

↓

Antenna / power / ground station / modulation choice